This read.me file covers installing the DigiBoard LAN Distance

driver on LAN Distance Servers and Remote Workstations.

- HARDWARE SUPPORTED

The driver supports the following DigiBoard Hardware products to

a maximum of 4 Host adapter cards and a maximun of 256 ports.

ISA Bus  :

PC/16i, PC/8i, PC/16e, PC/8e, PC/4e, PC/2e, PC/Xem, and C/X.

EISA Bus :

PC/16i, PC/8i, PC/16e, PC/8e, PC/4e, PC/2e, PC/Xem, EISA/Xem,

C/X and EISA C/X.

MCA Bus  :

MC/16i, MC/8i, MC/4i, MC/8e, MC/4e, MC/2e, MC/Xem, MC C/X

- HARDWARE INSTALLATION

Hardware installation should be done in accordance with the DigiBoard

Hardware Installation manual that you received with your DigiBoard

product.

You should choose a memory address and I/O port that does not conflict

with other devices in your system. You may address ISA and MCA bus

products up to the 16 Megabyte boundary if your computer does not

have system memory installed that high. On EISA computers, memory

addresses may be selected through the 4th Gigabyte range.

On MCA computers beware of configuring I/O ports in the 100 - 10f

range as they may conflict with POS I/O space.

- SOFTWARE INSTALLATION

The installation process consists of

1) Copying files to your \IBMCOM\MACS directory.

2) Automatically Updating your CONFIG.SYS

3) Installing support for the DigiBoard driver under LAPS.

4) Configuring the LAN Distance product to use the DigiBoard ports.

5) Manually Updating your CONFIG.SYS

* STARTING THE INSTALL PROCESS

This process will begin when you run INSTALL.CMD found on the Digi-

Board driver diskette. Insert the diskette into a floppy drive on

your system and make that drive the current drive. Then simply type

INSTALL < LAN Distance Drive: >  [< CONFIG.SYS Drive: >]

where

< LAN Distance Drive: > is the drive LAN distance is

installed on.

[< CONFIG.SYS Drive: >] is the drive where your CONFIG.SYS

is located. If it is the same a the LAN Distance Drive,

you may omit this parameter.

* Step 1 : COPYING FILES

Several files will be copied to your C:\IBMCOM\MACS directory. These

files are as follows :

DGAFGP.OS2

DGPOLLER.EXE

*.BIN

* Step 2: AUTOMATICALLY UPDATING CONFIG.SYS

The next action that the INSTALL.CMD script will perform is to

update your CONFIG.SYS with the following lines :

rem DigiBoard PC/16e, PC & MC/8e, 4e, 2e, and PC/Xi

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:320

rem DigiBoard MC/16i, MC/8i, MC/4i

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:C0000 /P:F1F0

rem DigiBoard PC/Xem, MC/Xem

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:324 /T:P

rem DigiBoard EISA Xem, slot 3

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:3005 /T:P

rem DigiBoard ISA C/X 1 concentrator

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:328 /T:I10

rem DigiBoard EISA C/X 1 concentrator, slot 3

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:3005 /T:E10

rem DigiBoard Micro Channel C/X 1 concentrator

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:328 /T:M10

RUN=C:\IBMCOM\MACS\DGPOLLER.EXE

Note : Several of the lines a remarked out and thus inactive. This

will be dealt with in the last part of the install process.

* Step 3 : LAPS INSTALLATION

Next, LAPS will be run to install the DigiBoard MAC driver so that

the DigiBoard ports will be available to LAN Distance. This will

require some minimal user interaction. When LAPS is run you will see

a LAPS screen with the following options :

Install     Configure     Remove     Exit    Help

* You should click on the "Install" option.

When you do, you will see a box titled "Install Additional Network

Drivers" and a dialog box where you should indicate the source drive.

* Enter the drive letter in which the DigiBoard Driver Diskette is

located and click on "OK".

At this point, more files will be transfered and you will be notified

that the installation is complete and that LAPS has successfully

installed the "DigiBoard Communication Ports".

* Click "OK" again.

You will return to the first LAPS screen you saw.

* Click "Exit"

You will see another dialog box titled "CONFIG.SYS Updates"

* Click "Exit" again.

You will see the final LAPS message informing you that you are

"Exiting LAPS".

* Step 4 : CONFIGURING THE LAN DISTANCE PRODUCT

After INSTALL.CMD finishes, you will be required to configure

LAN Distance for the DigiBoard Ports you wish to use. The

configuration process is virtually the same for a Server or a

Remote Workstation.

* Start LAN Distance

* From the "LAN Distance - Workstations" Box

Select "Selected" from the menu bar.

Select "Open As ->".

Select "Settings".

* From the "MyWorkStation - Settings" Box

Select "Ports" Tab.

Push   "Add..." Button.

Highlight "OEM DigiBoard Communications Ports".

If this option is not shown, you have not properly

installed the DigiBoard Driver using LAPS in Step 2.

You should go to an OS/2 command prompt and run

\ibmcom\laps, or click on the "OS\2 System" icon

and run LAPS from the "System Setup" folder.

Push   "OK..." Button.

* From the "DigiBoard Communications Ports - Settings" Box

Push   "LAPS" Button.

* You are now in the "Configure Workstation" Box in the "LAN Adapter

and Protocol Support Settings" window.

Scroll Down the "Network Adapters" List and highlight the

"DigiBoard Communications Ports" selection.

Press the "Add" button.

Make sure that the "DigiBoard Communication Ports..." entry

has been added and highlighted in the "Current

Configuration" window.

Press the "Edit" button.

Enter a new Portname if the default name "DIGI1" is not to

your liking. If there is an error with the name you

choose, LAPS will warn you. Be sure to use different

names if adding more than one port.

Press the "OK" button when finished.

You must repeat this process for each DigiBoard Port you wish

to add.

Press the "OK" button to exit.

If you are configuring a LAN Distance Server you may get an

error message upon attempting to exit. The message  will say

something like

"All LAN Adapter and Protocol Support combinations

in the Current Configuration must contain at least

one protocol driver."

Click on the "OK". Most likely, the problem lies with the

entry that looks like this :

LAN Distance Logical Adapter ....

[BRIDGEFH]....

1 - [SR_BRIDGE]

You may highlight these lines one at a time starting with

the last line and press the "Remove" button. These are

entries in your \ibmcom\protocol.ini file that are inserted

by LAN Distance but apparently not liked by LAPS. Removing

them will cause no harm because LAN Distance will reinsert

them before you exit. Now you may press the "OK" button to

exit without further ado.

* Now, back in the "DigiBoard Communications Ports - Settings" Box,

Double-Click on the box's upper left hand corner button to

close it.

* Now, you should be back in the "MyWorkStation - Settings" Box and

should see the "DigiBoard Communication Ports" entry in the

"Configured ports" window. If this is the case, you have success-

fully added your DigiBoard ports.

* You are now ready to complete the LAN Distance configuration to your

liking. This will include assigning modems to each port you added,

as well as setting up "Dial" and "Answer" parameters and making

"Phone Book" entries using the "MyWorkstation - Setting" tabs.

Please refer to your LAN DIstance documentation for information on

these topics.

****

When you have finished configuring LAN Distance, you MUST shut down

LAN Distance and save the settings.

****

* Step 5 : MANUALLY UPDATING CONFIG.SYS

Finally the time has come to update your CONFIG.SYS file to indicate

the type of DigiBoard you will be installing. Using your favorite

editor, bring up your CONFIG.SYS file. Near the bottom of the file

you will see the following entries that have been made.

rem DigiBoard PC/16e, PC & MC/8e, 4e, 2e, and PC/Xi

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:320

rem DigiBoard MC/16i, MC/8i, MC/4i

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:C0000 /P:F1F0

rem DigiBoard PC/Xem, MC/Xem

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:324 /T:P

rem DigiBoard EISA Xem, slot 3

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:3005 /T:P

rem DigiBoard ISA C/X 1 concentrator

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:328 /T:I10

rem DigiBoard EISA C/X 1 concentrator, slot 3

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:3005 /T:E10

rem DigiBoard Micro Channel C/X 1 concentrator

rem DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /M:D0000 /P:328 /T:M10

RUN=C:\IBMCOM\MACS\DGPOLLER.EXE

DEVICE=C:\IBMCOM\MACS\DGANDIS.OS2

Your task will be to uncomment and modify one of the "rem"ed "DEVICE="

lines to indicate the type and configuration of the DigiBoard you will

be installing. The lines that have been added to you CONFIG.SYS are

examples of typical configurations for the various DigiBoard products

supported by the LAN Distance Driver. One of these entries may very

well match the configuration you are using, in which case you need to

just delete the "rem" statement in front of that entry. Once you are

satisfied with your entry, you may delete the other entries or you

may leave them as is for future reference.

Below is information on how to determine the entry you need to make :

All DigiCHANNEL intelligent asynchronous serial communications boards

must be installed from a single command line, which contains complete

set-up information for each board.  The format of the command line is:

DEVICE=[d:][Path]DGAFGP.OS2 /a:a /m:mem /p:port /t:card

[,/m:mem /p:port /t:card ]

[, ...]

The above command line must appear on a single command line in the

CONFIG.SYS file.  This line may be up to 255 characters long.

* Global Parameter.

This type of parameter affects all boards installed. These parameters

are optional, and may be omitted. If omitted, default values will be

used.

- Parameter /a:a

Enable the alternate pin assignment of Data Carrier Detect

(DCD) for all ports. Alt-Pin is a feature that swaps the

meaning of the DSR and DCD input pins on each DigiBoard port.

This is useful when using an 8-pin RJ-45 plug in a DigiBoard

10-pin RJ-45 jack. In this configuration, pins 1 and 10 in

jack are not accessible to the 8-pin plug. Pin 10 is DCD and

very useful when attaching to a modem. Enabling the Alt-Pin

option causes the DigiBoard driver to interpret pin 2

(usually DSR) as DCD and pin 10 (usually DCD) as DSR. In

order to use this feature, your cables must be made to run

the real DCD signal into pin 2 of the DigiBoard jack (pin 1

on the 8-pin plug).

* Per-Card Parameters.

The remaining three parameters are per-card parameters, and are used

to inform the driver of the type of board and where it is to be

addressed in the hosts I/O and memory buses.  These parameters are

mandatory, and must be given for each board!  The per-card parameters

must be given in complete sets, and the sets must be separated by

commas.

- Parameter /m:mem

Dual-ported memory starting address.  This is the full

32-bit hexadecimal address of the beginning of the boards

dual-ported memory.

/m:d0000      Sets dual-ported memory start address to

0D0000h (in the BIOS expansion area between

640K and 1 megabyte).

/m:e80000     Sets start address of E80000h (in the

fifteenth megabyte).  EISA and ISA boards

may all share the same memory starting address.

Micro Channel boards must each have unique

memory addresses.

- Parameter /p:port

Defines the I/O port address for the card.

/p:300        Board is addressed at I/O port 300h.

/p:2005       EISA board (EISA C/X or EISA/Xem host adapter)

is in slot 2 (EISA I/O addresses are 4

digits : the slot number, followed by 005).

When plugging an ISA board (PC/Xe, ISA

C/X host adapter, etc.) into an EISA

machine, the /p parameter must contain the

3-digit I/O address as set by the DIP switches

on the board, and not the 4-digit address

based on the EISA slot number.

- Parameter /t:card

Defines the card type to the driver.  Legal values for card are:

i for the ISA C/X,

e for the EISA C/X,

m for the MC C/X,

p for Xem (PC/Xem, MC/Xem or EISA/Xem)

This parameter must be omitted for PC/Xe, PC/Xi, MC/Xe and MC/Xi

boards.  The designators i, e, and m must be followed by an

additional two digits defining the number and arrangement of

DigiCHANNEL C/CON-16 or C/CON-16e concentrators on each host

adapter channel. If the concentrator(s) are connected to the

host adapter via a dial-up or leased line (remote concentra-

tors), each of the two digits must be followed by a parameter,

enclosed in parentheses, specifying the synchronous commun-

ications mode to be used for that line.  See below for a list of

available synchronous communications modes.

/t:i10        ISA C/X, one local concentrator on line 1 only.

/t:e11        EISA C/X, one local concentrator on each of

lines 1 and 2.

/t:m21        MC C/X, two local concentrators on line 1 and

1 on line 2.

/t:e12        EISA C/X, one local concentrator on line 1,

and two on line 2.

/t:i22        ISA C/X, two local concentrators on each of

lines 1 and 2.

/t:p           PC/Xem, MC/Xem or EISA Xem

/t:i2(21)3(27)

ISA C/X, two remote concentrators on line 1,

and two remote concentrators on line 2.

The concentrators on line 1 will use mode

#21 (76,800 baud, 8-wire, external clock) to

communicate with the host adapter, and the

concentrators on line 2 will use mode #27

(614,400 baud, 8-wire, internal clock).  See

below for a list of synchronous communications

modes.

C/X Synchronous Communicatio Modes

----------------------------------------------------

|     8-Wire           8-Wire          4-Wire      |

| Internal Clock   External Clock    Self-Clock    |

----------------------------------------------------

|  Mode   Baud       Mode   Baud      Mode   Baud  |

|---------------------------------------------------

|                                                  |

|    0    115k        15    2400       1     230k  |

|                                                  |

|    3    2400        16    4800       2     460k  |

|                                                  |

|    4    4800        17    9600                   |

|                                                  |

|    5    9600        18    19.2k                  |

|                                                  |

|    6    19.2k       19    38.4k                  |

|                                                  |

|    7    38.4k       20    57.6k                  |

|                                                  |

|    8    57.6k       21    76.8k                  |

|                                                  |

|    9    76.8k       22    115k                   |

|                                                  |

|    10   115k        23    230k                   |

|                                                  |

|    11   230k        24    460k                   |

|                                                  |

|    12   460k        25    920k                   |

|                                                  |

|    13   920k        26    1.2M                   |

|                                                  |

|    14   1.2M                                     |

|                                                  |

|                     29    614k                   |

|                                                  |

|                     30    737k                   |

|                                                  |

|    27   614k                                     |

|                                                  |

|    28   737k                                     |

|                                                  |

----------------------------------------------------

* Examples

The following examples start out with just the minimum to install a

DigiCHANNEL board.  After locating an example for your type of board,

read the rest of the examples for optional situations that may be

appropriate to your application.  Default options will be altered by

LAN DIstance.

Example 1:

PC/Xe, PC/Xi One PC/Xe (or PC/Xi) at I/O port 220h, memory address

D0000h.

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /m:d0000 /p:220

Example 2:

MC/Xi One MC/Xi at I/O port F1F0h, memory address FC0000h.

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /m:FC0000 /p:F1F0

Example 3:

Two board installation with Alt-Pin optional global parameter.

Two PC/Xe or PC/Xi boards at I/O ports 320h and 300h, with

dual-ported memory of both starting at D0000h.

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2  /a:a /m:D0000 /p:320,

/m:D0000 /p:300

(Note that the above must all appear on one line in CONFIG.SYS)

Example 4:

Two board installation with Alt-Pin optional global parameter.

One MC/Xe and one MC/Xi boards at I/O ports F1F0h and 320h, with

different (can't be the same) dual-ported memory addresses. Alt-Pin

enabled.

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2  /a:a /m:C0000 /p:F1F0,

/m:C8000 /p:300

(Note that the above must all appear on one line in CONFIG.SYS)

Example 5:

PC/Xem One PC/Xem at I/O port 224h, memory start address C8000h;

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /m:C8000 /p:224 /t:p

Example 6:

ISA C/X One ISA C/X host adapter at I/O port 228h, memory address

D0000h, with one concentrator on line 1 and two on line 2.

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /m:d0000 /p:228 /t:i12

Example 7:

EISA C/X with a remote concentrator via an RS-232 synchronous modem.

One EISA C/X host adapter in slot 3, memory address D8000h, with two

local concentrators on line 1 and one remote concentrator at 9600

baud and external clocking on line 2.

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /m:d8000 /p:3005 /t:e21(17)

Example 8:

MC C/X and MC/Xem One MC C/X host adapter at I/O port 228h, memory

address D8000h, with one concentrator on line 1 and one concentrator

on line 2; one MC/Xem at I/O Port 304h, memory address D0000h.

Alt-Pin enabled

DEVICE=C:\IBMCOM\MACS\DGAFGP.OS2 /a:a /m:d8000 /p:228 /t:m11,

/m:d0000 /p:304 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

- DRIVER LOAD

As you may have noticed from the CONFIG.SYS file, the DigiBoard LAN

Distance driver is made up of 2 drivers. One driver that interacts

with the Protocol driver (DGANDIS.OS2) and one driver that interacts

with the hardware (DGAFGP.OS2). So when you boot your system, you

will see two drivers load. The order in which they load will follow

their order in the CONFIG.SYS file and should be DGAFGP.OS2 followed

sometime thereafter by DGANDIS.OS2, however, they can be loaded in

either order.

Loading DGAFGP.OS2 -

When this driver loads, it's primary goal is to find the DigiBoard

hardware specified on the "DEVICE=" line in CONFIG.SYS, initialize

it, and assure that it is functional. The driver displays the banner

shown below when the driver loads successfully.

-----------------------------------------------------------

| DigiBoard ANDIS Intelligent Board Driver        V 0.0.1 |

| DGAFGP.OS2   Copyright (C) 1994                         |

| All Rights Reserved.                                    |

-----------------------------------------------------------

-----------------------------------------------------------

|  RESTRICTED RIGHTS LEGEND:                              |

|  Use, duplication, or disclosure by the Government is   |

|  subject to restrictions set forth in sub-paragraph (c) |

|  (1) (ii) of the Rights in Technical Data and Computer  |

|  Software clause at DFARS 252.227-7013.                 |

|                                                         |

|  Digi International Inc. d/b/a  DigiBoard, 6400 Flying  |

|  Cloud Drive, Eden Prairie, MN 55344                    |

-----------------------------------------------------------

-----------------------------------------------------------

| Board Type   I/O Address    Memory Address     Channels |

-----------------------------------------------------------

PC/Xem          324          0x000D0000           16

If there are any errors encountered as the driver loads, error

messages will be posted to alert you of the condition and OS/2

should stop the boot process and prompt you to continue.

Possible error messages include :

* CONFIG.SYS errors

"Incomplete specification: need IO port AND memory for each board."

"Unexpected character: "

"Too many boards: Only 4 Intelligent boards supported."

"Unknown value assigned"

"Multiple IO port specifications: (/p:_)"

"Duplicate IO port address specifications: (/p:_)"

"Invalid IO port address specification: (/p:_)"

"Multiple memory address specifications: (/m:_)"

"Invalid memory address specification: (/m:_)"

"Invalid board type code: (/t:_)"

"No DigiBoards Specified on command line"

"Invalid Command Line Arguments"

"Zero DigiBoards Configured"

"Too many channels: maximum = 256"

- Cause & Remedy

The driver has encountered the specified error as it was parsing the

CONFIG.SYS "DEVICE= line". You should check your CONFIG.SYS for

accuracy.

* Hardware Errors

"Failed to RESET the I/O PORT"

"IO Port Not Responding"

"Failed Shared Memory Test near OFFSET: 0xXXXX"

"Failed to write to shared memory"

- Cause & Remedy

There is a memory or I/O port conflict with an other device in your

system or you have a defective board. Try a different I/O port or

memory address.

* FEP and BIOS Binary file problems

"Invalid DATA FILES DIRECTORY"

"Failed to locate data files"

"Failed to STAT file"

- Cause & Remedy

These messages usually occur while the driver is attempting to locate

and open the *.bin files for download to the DigiBoard card. First,

make sure that the *.bin files on your driver diskette have been

copied to the \ibmcom\macs directory. If they are there, you may need

to recopy them from the driver diskette in case a file was corrupted.

If this fails to remedy th condition, you may need to add another

command line argument to your "DEVICE=" statement in the CONFIG.SYS

file. The argument is " /f:<path to *.bin files> ". This tells the

driver where the files are located. If the errors persist, you may

have a conflict with the DigiBoard hardware and your disk controller.

Move the DigiBoard to another I/O port and memory address.

* DigiBoard Initialization Errors

"Failed to BOOT the BIOS"

"Failed to BOOT the uploaded FEPOS"

"Failed to Read Initialization Information"

"DigiBoard FEPOS will not execute commands"

"DigiBoard BIOS Will Not Boot"

"DigiBoard BIOS Will Not Respond"

"DigiBoard FEPOS Will Not Boot"

"Unknown Fatal Error"

- Cause & Remedy

These messages usually indicate problems initializing the card's on-

board software and are usually the results of a memory conflict with

another device in your system, a corrupt *.bin file, or a faulty

board. First, recopy the *.bin files from your driver diskette and

reboot your system. If this doen not work, relocated your board to

another I/O port and/or memory address. If none of this works, you

may have a bad board.

* OS/2 System Errors

"System Cannot Allocate Memory"

"Driver Failed to hook Timer",

"System Cannot Allocate Ctx Hook",

"Failed to make upload FILE memory"

- Cause & Remedy

Chances are your system resources are depleted. Try unloading any

unnecessary drivers to see if the problem goes away. If this doesn't

work, get a bigger computer!

* Possible Hardware problems

"Zero channels installed."

- Cause & Remedy

This usually means that the card's on-board code could not locate

any ports. Usually you will see this message with the Xem and C/X

products when you forget to connect the EBI or concentrators. Check

this to see if that may be the case. Other wise you may have a faulty

card.

Loading DGANDIS.OS2 -

When this driver loads, it's primary goal is to determine the LAN

Distance ports you have configured using LAPS, and register these

with the protocol driver. The driver internal tables and memory

are also allocated and initialized. The driver displays the banner

shown below when the driver loads successfully.

-----------------------------------------------------------

| DigiBoard Intelligent Board ANDIS MAC Driver    V 0.0.1 |

| DGANDIS.OS2   Copyright (C) 1994                        |

| All Rights Reserved.                                    |

-----------------------------------------------------------

Installing DGANDIS

Initializing port: DIGI1

Initializing port: DIGI2

DGANDIS is installed

If the driver encounters an error while loading, one of the following

messages will be displayed. Since this driver does not initialize

any hardware the sources of error are usually software in nature

and fairly difficult for you to remedy but some suggestions may help.

* Driver Registration Problems

"Can not open PROTMAN"

"PROTMAN returns error on config read"

"Register port to PROTMAN returns Failure"

"DosDevIOCtl Fails on register to PROTMAN"

"PROTMAN does not support PPAT"

"PPAT registration Failed"

"DosDevIOCtl Failed on GetPMInfo"

- Cause & Remedy

PROTMAN.OS2, the protocol manager, and the driver are having some

interaction difficulties. First, check to see that a

"DEVICE=PROTMAN.OS2"

statement is in your CONFIG.SYS file. Try recopying the DGANDIS.OS2

driver from the driver diskette. Also, you may have a wrong version

PROTMAN.OS2.

* PROTOCOL.INI Problems

"ProcessKeyWords Failed"

- Cause & Remedy

Check to see that your \ibmcom\protocol.ini file has a "PORTNAME="

line under [DGANDIS_nif] entry.

* OS/2 System Errors

"DGANDIS Failed to install"

"Internal Error"

"AllocPhysMem Failed"

"AllocGDT Failed"

"Phys2GDT Failed"

"VirtualAddr Failed"

"AllocSpareGDT Failed"

- Cause & Remedy

Chances are your system resources are depleted. Try unloading any

unnecessary drivers to see if the problem goes away. If this doesn't

work, get a bigger computer!