Daniela's OS2LDR memory detection patch

---------------------------------------

SYNOPSIS

Many (most/all?) recent motherboards got a BIOS which causes OS/2 to

fail proper detection of the size of installed memory. The PatchLDR

utility fixes that.

DESCRIPTION

From Warp 4 on, OS/2 uses the BIOS function Int15 [AX=E801] to get the

size of installed memory. In the recent past some authority sentenced

this function as 'not ACPI compliant' and made the BIOS manufacturers

to remove this function. WinXX is not affected by that change because

it uses BIOS function Int15 [EAX=0000E820] which was added later and which

is considered 'ACPI compliant'.

You may run the DOS based utility MEMTEST from this package to determine if

your BIOS exhibits the defect I described. Straight DOS is preferred, but

it works in an OS/2 VDM sufficiently well. If BIOS function Int15 (E801) is

reported as 'not supported', and you have more than 64 MiB of memory

installed, the PatchLDR utility will solve the problem. The return code

of MEMTEST will tell you as well if you need the patch:

rc =  0 : patch is not required

rc =  1 : patch method E820 is recommended

rc =  2 : patch method DA88 is recommended

The utility patches OS2LDR so that the query for the size of installed

memory no longer uses BIOS function Int15 [AX=E801] but

Int15 [EAX=0000E820]. Due to space constraints I had to remove the

memory detection routines for PS/2 machines. But, as these aren't

affected by the recent BIOS changes, this is a non-issue.

***  This method now supports non-contiguous memory above 1MiB as well, if the

discontinuity is below the 16MiB line. This is found with boards that have

a memory hole between 15MiB and 16MiB enabled or suffer from a defect

in the ACPI memory table.

***  There are broken BIOS implementations which report back wrong values for

functions other than Int15 [EAX=0000E820]. This can be worked around with

'PatchLdr E820' as well.

Another choice is to use BIOS function Int15 [AX=DA88] instead of

BIOS function Int15 [AX=E801].

This patch utility is supposed to work with all fixpacks of Warp 4, the

later fixpacks of Warp 3 (including derivatives), and the current versions

***  of Warp Server for e-Business, MCP, ACP, and eCS (including fixpacks and

***  updated kernels). It was able to patch all of my different Warp 3/4/WSeB

installations successfully. You may decide to run OS/2 with the patched

OS2LDR even if you don't need it right now.

According to my own tests and a lot of user reports it solves the memory

size defect found with all (?) of the Athlon boards and others as well. So

I consider the patch as safe.

USAGE

1) move to the root of the boot partition.

2) enter the command 'ATTRIB -R -S -H OS2LDR'.

3) enter the command 'PatchLDR' or 'PatchLDR E820' or 'PatchLDR DA88'.

If step 3 fails, an error message will be displayed. If it succeeds, the

old OS2LDR is copied to OS2LDR.bak, and a new, patched OS2LDR is created.

4) enter the command 'ATTRIB +R +S +H OS2LDR'.

From the next reboot on, the new memory size detection code is in effect.

Please remember: after installing a fixpack the OS2LDR patch needs to

be applied again because it will be replaced by a non-patched version

from the fixpack. If the fixpack installer asks you if it should replace

the patched version of OS2LDR with the one from the fixpack distribution,

then reply 'YES'! Failing that may render OS/2 unbootable because

OS2LDR and OS2KRNL need to match.

Reminder: as patching OS2LDR doesn't affect your BIOS in any way, MEMTEST

will show exactly the same output after applying the patch.

This is correct!

COPYRIGHT

MEMTEST and PatchLDR - Copyright (c) 1999,2000,2001 Daniela Engert.

All rights reserved.

STATUS OF PROGRAM

MEMTEST and PatchLDR are freeware.

DISCLAIMER

YOU ARE USING THIS PROGRAM AT YOUR OWN RISK! I don't take any

responsibillity for damages, problems, custodies, marital dis-

putes, etc. resulting from use, inability to use, misuse, pos-

session or non-possession of this program directly or indirectly.

I also don't give any warranty for bug-free operation, fitness for

a particular purpose or the appropriate behaviour of the program

concerning animals, programers and little children.

THE SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS OR

IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE

PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE DEFECTIVE, YOU

ASSUME THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

Or, in a few words:

If its good, I am responsible.

If its bad, its all your fault. ;)

Permission is granted to redistribute this program free of charge,

provided it is distributed in the full archive with unmodified

contents and no profit beyond the price of the media on which it

is distributed is made. Exception to the last rule: It may be

included on freeware/shareware collections on CD-ROM, as well as

on magazine cover CD-ROMs.

All trademarks mentioned anywhere around her are property of their

owners and the like ...

AUTHOR

Daniela Engert

Internet: dani@ngrt.de

Fidonet: 2:2490/2576

EPILOGUE

Suggestions and bug-reports are always welcome.